产品规格书

PRODUCT SPECIFICATION

产品名称：

PRODUCT　 　NAME：               4*8胶壳保险丝   FUSE

型号规格：

MODEL DETAIL：          GMT  T1A  400V   延时型(TIME-LAG)

料  号：

PART NO.：                           GMT1100D

| 编 制 PREPARED BY | 审 核 CHECKED BY | 批 准 APPROVED BY |  |
| --- | --- | --- | --- |

1. 适用范围Scope of application

本产品适用于各种电子机器内的电路中起过电流保护作用.

This product is suitable for various kinds of electronic machines to work for over current protection.

2.相关标准及认证Relevant Standards and Certification

2.1执行标准Standards：UL148-1

2.2认证Certification：

| 认证 Certification | 认证范围 Scope of Certification | 额定电压 Voltage Rating | 证书编号 Certification No. |
| --- | --- | --- | --- |
| UL | 50mA~10A | 400V | E493749 |

3. 标示 Marking

保险丝应有下列标示The fuse shall have the following marking :

巨凌标志

延时符号 Time-lag Symbol：T

额定电流 Ampere rating：1A

额定电压 Voltage rating：400V

认证符号 Approved Symbol：

4.结构及尺寸Structure and Size (unit: mm)

5.原材料明细 Material Details

| 编号 No. | 零件名称Part Name | 材质 Material |
| --- | --- | --- |
| ① | 外壳 Case | Thermoplastic Nylon（UL94-V0） |
| ② | 基座 Base | Thermoplastic Nylon（UL94-V0） |
| ③ | 熔丝Fuse element | 合金线Alloy |
| ④ | 引线Lead wire | 镀锡铜线Tinned copper wire |

6. 外观Appearance

6.1  外观不应有显著的污点、裂纹。

There shall not be any remarkable stain，rust or crack on the appearances.

6.2  标示应该很容易辨识。

Marking shall be easily legible.

7. 机械特性Mechanical characteristics

拉力强度试验Tensile test

将保险丝固定好后，对保险丝引线施加10N的拉力，持续1分钟。试验过后，保险丝没有任何破损现象。

Exert the axial pulling force of 10N to the both ends of lead wire for 1 minute. After the test the fuse shall be no damage and loosening.

8. 电气特性Electrical characteristics

8.1  实验条件Test condition

所有测试环境温度为25±5℃。

All electrical characteristics tests are conducted at an ambient temperature of 25±5℃.

8.2  分断能力Interrupting rating test

保险丝能承受的分断能力：35A或10倍的额定电流当中较大的。分断能力测试后测得保险

丝的绝缘阻抗值大于0.1ΜΩ。

The fuse interrupting capacity：35A or 10 times the rated current whichever is greater. After

interrupting, the insulation resistance between the fuse terminations shall be not less than 0.1ΜΩ.

8.3 时间-电流特性 Time-Current Characteristics

| 额定电流 Ampere Rating | 额定电流百分比 % of Ampere Rating |  |  |  |
| --- | --- | --- | --- | --- |
|  | 210% | 275% | 400% | 1000% |
| 0.5A-6.3A | 2 min Max | 400Ma-10s | 150ms-3s | 20ma-150ms |
| 7A-10A | 2 min Max | - | - | 20ma-150ms |

8.4 耐久性测试 Endurance test

对保险丝通过相关标准规格单规定的额定电流1小时,然后截断电流15分钟,将该循环重复

100次。试验过后保险丝没有任何破损现象。

A ampere rating specified in the relevant standard sheet is passed through the fuse for a period of

1h.The current is then switched off for a period of 15 minutes. This cycle is repeated 100 times. After

the test the fuse shall be no damage and loosening.

8.5 焊接参数 Soldering parameters

8.5.1 波峰焊 260℃,最大10秒，

Wave soldering 260 ℃,10 sec. Maximum

8.5.2 手工焊 370℃,最大5秒，

Manual soldering 370℃, 5 sec. Maximum

8.6 时间电流曲线图见附页 Time- current curve see attachment

9. 环境特性Environmental characteristics

9.1使用温度范围 Operating Temperature

-55℃～125℃。

9.2保存条件Storage Conditions

相对湿度≤75%，可存放3年。

Relative humidity≤75%, store 3 years .

10. 包裝Packing

10.1 自动机器编带1000PCS/内盒、10内盒/外箱(实际装箱依客户订单)。

Automaton taped 1000 PCS per inner box, 10 inner boxes per external carton. (The actual case in

accordance with the customer order)

10.2 产品符合RoHS环保指令

The product is compliant with RoHS Directive

附页 attachment